Automatic Door Service Record

An automatic door and gate service record: each door with type and label scan, activation and safety sensors, obstruction stop, manual opening on power failure, emergency break-out, signage, result and recommendations.

1. Visit

Project

______________________________________________________________________________

Client

______________________________________________________________________________

Site *

______________________________________________________________________________

Site address

______________________________________________________________________________

Service date *

______________________________________________________________________________

Time

______________________________________________________________________________

Engineer *

______________________________________________________________________________

Company

______________________________________________________________________________

Workplace (Health, Safety and Welfare) Regulations 1992, regulation 18: a powered door or gate must have suitable and effective features to prevent it causing injury by trapping anyone, and where needed for health or safety it must be possible to open it by hand unless it opens automatically if the power fails. HSE's Approved Code of Practice (L24) gives examples: a sensitive edge or other detector that stops or reverses the door when obstructed, and a device that limits the closing force.

2. Doors and gates

Doors  (copy this block for each one)

Location *

______________________________________________________________________________

Type  (choose one)

☐ Sliding door

☐ Swing door

☐ Revolving door

☐ Folding door

☐ Sliding gate

☐ Swing gate

☐ Barrier

Scan the operator label

Scanned in the SiteSurvey app. On paper, write the numbers: serial number, model number.

______________________________________________________________________________

______________________________________________________________________________

Activation sensors and controls  (choose one)

☐ OK     ☐ Defect

Safety sensors stop or reverse the door on an obstruction  (choose one)

☐ OK     ☐ Defect

Closing force and speed within limits  (choose one)

☐ OK     ☐ Defect

Closing force measured with a force meter?

☐ Yes     ☐ No

Opens by hand on power failure, or opens automatically  (choose one)

☐ OK     ☐ Defect

Emergency break-out or escape function  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

Signs and markings  (choose one)

☐ OK     ☐ Defect

Result *  (choose one)

☐ Safe     ☐ Safe with recommendations     ☐ Unsafe: taken out of service

Defects and recommendations  (if applicable)

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

If applicable: Unsafe. Leave it out of service and tell the site contact in writing.

Who was told it is out of service, and when  (if applicable)

______________________________________________________________________________

Photos  (photos: attach, or note the file names)

______________________________________________________________________________

Doors taken out of service  (worked out automatically in the app)

______________________________________________________________________________

Doors with recommendations  (worked out automatically in the app)

______________________________________________________________________________

3. Sign-off

Site contact name

______________________________________________________________________________

Site contact signature

Completed by *

______________________________________________________________________________

Signature