Bathroom Survey

A bathroom measure survey: room size and height, every wall with its features, window and door, existing suite, water, waste, soil stack and electrics, extraction, floor, the new layout wanted and the customer's signature.

1. Room

Project

______________________________________________________________________________

Client

______________________________________________________________________________

Site *

______________________________________________________________________________

Site address

______________________________________________________________________________

Survey date *

______________________________________________________________________________

Time

______________________________________________________________________________

Surveyed by *

______________________________________________________________________________

Room length (mm) *

______________________________________________________________________________

Room width (mm) *

______________________________________________________________________________

Ceiling height (mm)

______________________________________________________________________________

Floor area (m²)  (if applicable; worked out automatically in the app)

______________________________________________________________________________

Floor  (choose one)

☐ Suspended timber     ☐ Concrete     ☐ Tiled on board     ☐ Other

Walk Cam: bathroom survey

Walk Cam video walk-through: available in the SiteSurvey app only.

2. Walls and existing suite

Walls  (copy this block for each one)

Wall (A, B, C clockwise from the door) *

______________________________________________________________________________

Length (mm) *

______________________________________________________________________________

On this wall  (tick all that apply)

☐ Window

☐ Door

☐ Radiator or towel rail

☐ Soil stack

☐ Boxing

☐ Sockets or switches

☐ Shaver point

Positions and sizes from the left corner (mm)

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

Photos  (photos: attach, or note the file names)

______________________________________________________________________________

Existing suite  (tick all that apply)

☐ Bath     ☐ Shower over bath     ☐ Shower enclosure     ☐ Basin     ☐ WC     ☐ Bidet

3. Services

Soil stack position and WC outlet

______________________________________________________________________________

Waste routes and falls

______________________________________________________________________________

Hot water  (choose one)

☐ Combi boiler     ☐ Unvented cylinder     ☐ Vented cylinder     ☐ Electric

Water pressure (bar)

______________________________________________________________________________

Is there an extractor fan ducted to outside?

☐ Yes     ☐ No

If applicable: Approved Document F sets a minimum extract rate for a bathroom in England of 15 l/s for an intermittent fan, or 8 l/s high rate on a continuous system. Plan a fan in the new layout.

Approved Document P: a new circuit, a new consumer unit, or an addition or alteration to a circuit in the special location around a bath or shower (up to 0.6 m from the edge of the bath or tray, and up to 2.25 m high) is notifiable. Like-for-like replacements, repairs and maintenance are not. Use a registered electrician or notify building control.

Any materials that might contain asbestos (textured ceiling or wall coating, old floor tiles, boxing or panels)?

☐ Yes     ☐ No

If applicable: Control of Asbestos Regulations 2012, regulation 5: an employer must not start work that could disturb asbestos without first assessing whether it is present, or presuming it is. HSE says buildings built after 2000 are unlikely to contain asbestos. Arrange a survey before work that disturbs these materials.

4. New layout

Wanted  (tick all that apply)

☐ Bath

☐ Walk-in shower

☐ Wet room

☐ Basin with vanity

☐ WC

☐ Towel rail

☐ Underfloor heating

☐ Shaver point

☐ Mirror cabinet

☐ Other: ____________

Tiles, style and accessibility needs

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

Budget

______________________________________________________________________________

Customer name

______________________________________________________________________________

Customer signature