Conveyor Belt Inspection

A belt conveyor inspection: whether it was running or isolated, belt surface, edges, tracking and joints, rollers and drums, spillage, guards, pull cords and emergency stops, result, photos and repair actions.

1. Conveyor

Project

______________________________________________________________________________

Client

______________________________________________________________________________

Site *

______________________________________________________________________________

Site address

______________________________________________________________________________

Inspection date *

______________________________________________________________________________

Time

______________________________________________________________________________

Inspector *

______________________________________________________________________________

Conveyor reference *

______________________________________________________________________________

Type  (choose one)

☐ Flat belt

☐ Troughed belt

☐ Incline or decline belt

☐ Modular plastic belt

☐ Other

State during the inspection *  (choose one)

☐ Running: checked from the walkway only     ☐ Stopped and isolated: locked off

Isolation point and personal lock  (if applicable)

______________________________________________________________________________

Do not remove a guard or reach in while the conveyor can move. HSE advises, in its quarries guidance, that cleaning and maintenance of conveyors must only be done when the conveyor is properly isolated and the isolator locked with a personal lock and key, and that pull wires are additional protection, not an alternative to fixed guards.

PUWER regulation 11 requires measures to prevent access to dangerous parts of machinery, with fixed guards where practicable, kept in efficient working order and good repair. Regulation 6 requires inspection at suitable intervals where equipment is exposed to deterioration, and HSE says the result should be recorded and kept at least until the next inspection.

2. Checks

Walkways and access clear  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

Belt surface: no cuts, tears or worn patches  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

Belt edges: no fraying  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

Belt runs centrally (tracking)  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

Belt joint or fasteners  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

Rollers and idlers turn freely, none seized  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

Head and tail drums  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

No spillage or build-up under the belt  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

Fixed guards in place at drives, drums and nip points  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

Pull cord, where fitted, runs head to tail and stops the belt  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

Emergency stop buttons  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

Isolation point clearly marked and lockable  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

If applicable: A missing or damaged guard. Keep the conveyor out of use until it is refitted or repaired: PUWER regulation 11 requires guards to be maintained in an efficient state and in good repair.

If applicable: PUWER regulation 16 requires readily accessible emergency stop controls where appropriate. Report a faulty stop or pull cord before the conveyor is used.

Photos (from outside the guards)  (photos: attach, or note the file names)

______________________________________________________________________________

Walk Cam: conveyor belt inspection

Walk Cam video walk-through: available in the SiteSurvey app only.

3. Actions and sign-off

Overall result *  (choose one)

☐ Safe to use     ☐ Safe to use: repairs booked     ☐ Out of use

Are further actions needed?

☐ Yes     ☐ No

Further actions  (copy this block for each one; if applicable)

What needs doing? *

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

Who will do it?

______________________________________________________________________________

Due date

______________________________________________________________________________

Status  (choose one)

☐ Open     ☐ In progress     ☐ Completed

Inspector name *

______________________________________________________________________________

Position or company

______________________________________________________________________________

Inspector signature

Signing records what was found on the day. It does not make unsafe equipment safe or replace a thorough examination, design check or service by a competent person.