Culvert and Trash Screen Inspection

A culvert and trash screen inspection from the bank: watercourse and water level, flow, screen and debris, headwall, wing walls, banks and outlet, debris cleared, photos, who was told and actions. Never enter a culvert.

1. Site

Project

______________________________________________________________________________

Client

______________________________________________________________________________

Site *

______________________________________________________________________________

Site address

______________________________________________________________________________

Inspection date *

______________________________________________________________________________

Time

______________________________________________________________________________

Inspector *

______________________________________________________________________________

Watercourse and culvert reference *

______________________________________________________________________________

Structure  (choose one)

☐ Culvert with trash screen

☐ Culvert, no screen

☐ Outfall

☐ Outfall with screen

Inlet position (GPS)

Optional. Mark the culvert inlet or screen from the bank, so the next inspector and the authority find it.

______________________________________________________________________________

Reason for the inspection  (choose one)

☐ Routine

☐ Before forecast heavy rain

☐ After a flood or storm

☐ After a report of a blockage

Inspect from the bank or a safe platform and do not enter the water. GOV.UK (Environment Agency): do not go inside a culvert, you could get trapped or poisoned by gases. If it needs clearing or repairing, contact your risk management authority for advice.

GOV.UK (England): you normally own the stretch of watercourse on or under your land, including a culvert, or up to its centre if it is on your boundary. Owners must let water flow naturally and should keep any trash screen, weir or other structure clear.

Weather capture

Digital form: use Record current conditions. On paper, record date/time, temperature (°C), wind (mph and direction), conditions and source. Keep observed conditions separate from forecast estimates.

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

2. Inspection

Water level  (choose one)

☐ Low     ☐ Normal     ☐ High     ☐ In flood

Flow through the culvert *  (choose one)

☐ Flowing freely     ☐ Slowed     ☐ Blocked or backing up

Height of the culvert opening (m)

Optional. From the asset record or drawing, soffit to bed, or measure from a photo taken square on from the bank (an estimate). Do not enter the water.

______________________________________________________________________________

Gap from the water to the top of the opening (m)

Optional. Measure from a photo taken square on from the bank, using a known size in the same plane such as the screen width or the opening height from the asset record. Do not enter the water.

______________________________________________________________________________

Opening above the water (%)  (if applicable; worked out automatically in the app)

______________________________________________________________________________

Channel upstream clear of fallen trees and large debris  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

Trash screen clear of debris  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

Share of the screen blocked (%)  (if applicable)

Your estimate from the bank, before any clearing.

______________________________________________________________________________

Clear when blocked by more than (%)  (if applicable)

Optional. From the owner's maintenance plan for this screen.

______________________________________________________________________________

Against the clearance trigger  (if applicable; worked out automatically in the app)

______________________________________________________________________________

Screen bars and fixings intact  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

Headwall and wing walls: no cracks or missing blocks  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

Banks: no erosion or collapse  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

Outlet clear and flowing  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

Safe access, handrails and platform  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

Checks OK  (worked out automatically in the app)

______________________________________________________________________________

Defects  (worked out automatically in the app)

______________________________________________________________________________

Not applicable  (worked out automatically in the app)

______________________________________________________________________________

Debris found  (tick all that apply)

☐ Branches

☐ Leaves and vegetation

☐ Litter and plastics

☐ Silt

☐ Large items

Was debris cleared during this visit?

☐ Yes     ☐ No

How it was cleared, and where the debris went  (if applicable)

______________________________________________________________________________

Debris removed (bags)  (if applicable)

______________________________________________________________________________

If applicable: The screen is blocked beyond the clearance trigger and was not cleared on this visit. Arrange clearance, and report it if it could cause flooding.

If applicable: A blocked culvert or screen can cause flooding. Report blockages that could cause flooding: GOV.UK says the risk management authority is the Environment Agency for a main river, or the lead local flood authority or internal drainage board for any other watercourse.

Photos (from the bank)  (photos: attach, or note the file names)

______________________________________________________________________________

Walk Cam: culvert and trash screen inspection

Walk Cam video walk-through: available in the SiteSurvey app only.

3. Actions and sign-off

Reported to (owner or risk management authority) and reference

______________________________________________________________________________

GOV.UK: find out which permissions you need before you maintain, repair, build or remove anything in or around a watercourse, including a culvert.

Are further actions needed?

☐ Yes     ☐ No

Further actions  (copy this block for each one; if applicable)

What needs doing? *

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

Priority  (choose one)

☐ Urgent     ☐ High     ☐ Medium     ☐ Low

Who will do it?

______________________________________________________________________________

Due date

______________________________________________________________________________

Status  (choose one)

☐ Open     ☐ In progress     ☐ Completed

Actions open or in progress  (if applicable; worked out automatically in the app)

______________________________________________________________________________

Next inspection due

Set by the owner's own inspection regime. Also inspect before forecast heavy rain and after a flood or storm.

______________________________________________________________________________

Days until the next inspection  (if applicable; worked out automatically in the app)

______________________________________________________________________________

If applicable: Check the date. The next date is before the date of this visit.

Inspector name *

______________________________________________________________________________

Position or company

______________________________________________________________________________

Inspector signature

Signing records what was found on the day. It does not make unsafe equipment safe or replace a thorough examination, design check or service by a competent person.