Fire Extinguisher Inspection Record

A fire extinguisher inspection record: each extinguisher with its ID, location, type and service dates, the monthly checks, overdue service and extended service warnings, and pass or fail.

1. Inspection details

Project

______________________________________________________________________________

Client

______________________________________________________________________________

Site *

______________________________________________________________________________

Site address

______________________________________________________________________________

Date of the check *

______________________________________________________________________________

Time

______________________________________________________________________________

Checked by, name and role *

______________________________________________________________________________

Responsible person for the premises

______________________________________________________________________________

Type of check *  (choose one)

☐ Monthly check by the premises

☐ Annual basic service by a competent person

☐ Extended service or overhaul

Home Office guidance. Carry out brief checks to make sure extinguishers remain serviceable; in normal conditions a monthly check should be enough. Maintenance by a competent person should be carried out annually.

If applicable: BS 5306-3 maintenance intervals. Every extinguisher has a basic service every year. Water-based and powder extinguishers also have an extended service every 5 years (every 10 years for primary sealed powder). Carbon dioxide extinguishers have an overhaul every 10 years.

In England and Wales the Regulatory Reform (Fire Safety) Order 2005, article 17, requires the responsible person, where necessary to keep people safe, to make sure equipment provided for fire safety, such as extinguishers, is subject to a suitable system of maintenance and is kept in an efficient state, in efficient working order and in good repair.

2. Extinguishers

Extinguishers checked  (copy this block for each one)

Extinguisher ID or number *

______________________________________________________________________________

Scan the service label

Scanned in the SiteSurvey app. On paper, write the numbers: serial number, asset number.

______________________________________________________________________________

______________________________________________________________________________

Location *

______________________________________________________________________________

Type *  (choose one)

☐ Water

☐ Water mist

☐ Foam

☐ Carbon dioxide (CO2)

☐ Dry powder

☐ Dry powder, primary sealed

☐ Wet chemical

☐ Lithium-ion battery (specialist)

☐ Fire blanket

Size (litres or kg)

______________________________________________________________________________

Year of manufacture

______________________________________________________________________________

Date of the last service (from the label)

______________________________________________________________________________

Days since the last service  (if applicable; worked out automatically in the app)

______________________________________________________________________________

If applicable: Service overdue. The service label is more than 12 months old. Arrange a service by a competent person.

Date of the last extended service or overhaul  (if applicable)

______________________________________________________________________________

If applicable: Extended service due. More than 5 years since the last extended service.

If applicable: Extended service due. More than 10 years since the last extended service of this primary sealed powder extinguisher.

If applicable: Overhaul due. More than 10 years since the last overhaul of this carbon dioxide extinguisher.

In its place, visible and easy to reach  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

Identification sign or label present  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

Safety pin and tamper seal in place  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

Pressure gauge in the green (stored pressure types)  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

No damage, dents, corrosion or leaks  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

Hose, horn and nozzle not cracked or blocked  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

Operating instructions readable and facing outwards  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

Bracket or stand secure, handle at a height that is easy to lift off  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

Has it been used or partly discharged?

☐ Yes     ☐ No

If applicable: Used or seal broken. Treat it as out of service. Replace it or have it recharged by a competent person, and record it in the fire log book.

Result *  (choose one)

☐ Pass     ☐ Fail

Photos of the fault  (if applicable; photos: attach, or note the file names)

______________________________________________________________________________

What was done, and what is still needed?  (if applicable)

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

Replacement or spare left in place  (if applicable)

______________________________________________________________________________

Extinguishers failed  (worked out automatically in the app)

______________________________________________________________________________

Extinguishers passed  (worked out automatically in the app)

______________________________________________________________________________

Were all extinguishers on the premises checked?

☐ Yes     ☐ No

Which were missed, and when will they be done?  (if applicable)

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

Has this check been entered in the fire safety log book?

☐ Yes     ☐ No

3. Actions and sign-off

Are further actions needed?

☐ Yes     ☐ No

Further actions  (copy this block for each one; if applicable)

What needs doing? *

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

Who will do it?

______________________________________________________________________________

Due date

______________________________________________________________________________

Status  (choose one)

☐ Open     ☐ In progress     ☐ Completed

Inspector name *

______________________________________________________________________________

Position or company

______________________________________________________________________________

Inspector signature

Signing records what was found on the day. It does not make unsafe equipment safe or replace a thorough examination, design check or service by a competent person.