Heat Pump Site Survey

A heat pump site survey: property, current heating and insulation, room-by-room measurements and emitters for the heat loss calculation, outdoor unit position, hot water cylinder space and electrics.

1. Property

Project

______________________________________________________________________________

Client

______________________________________________________________________________

Site *

______________________________________________________________________________

Site address

______________________________________________________________________________

Survey date *

______________________________________________________________________________

Time

______________________________________________________________________________

Surveyor *

______________________________________________________________________________

Property type  (choose one)

☐ Detached house

☐ Semi-detached house

☐ Terraced house

☐ Bungalow

☐ Flat

☐ Other

Age of the property  (choose one)

☐ Before 1919

☐ 1919 to 1944

☐ 1945 to 1979

☐ 1980 to 1999

☐ 2000 or later

☐ Not known

Bedrooms

______________________________________________________________________________

People living there

______________________________________________________________________________

Current heating *  (choose one)

☐ Mains gas     ☐ Oil     ☐ LPG     ☐ Electric     ☐ Solid fuel     ☐ Other

Boiler Upgrade Scheme (England and Wales). £7,500 towards an air source or ground source heat pump replacing fossil fuel heating, for the owner of the property, with an MCS certified installer applying on their behalf. Until March 2027 there is an extra £1,500 if the property is heated by oil or LPG and has no mains gas connection. Hybrid systems are not covered.

EPC rating, if known

______________________________________________________________________________

2. Insulation and fabric

Main wall type  (choose one)

☐ Cavity, filled

☐ Cavity, unfilled

☐ Solid

☐ Timber frame

☐ System built

☐ Not known

Loft insulation depth (mm)

______________________________________________________________________________

Glazing  (choose one)

☐ Single     ☐ Double     ☐ Triple     ☐ Mixed

Ground floor  (choose one)

☐ Solid     ☐ Suspended timber     ☐ Not known

Draughts, damp or other things that affect heat loss

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

3. Rooms

MCS installers design heat pumps to MIS 3005-D, the MCS heat pump design standard. It calls for a heat load calculation to BS EN 12831-1 using internal design temperatures no lower than its table (for example living room 21°C, bedroom and kitchen 18°C, bathroom 22°C), and the installer is given the heat loss of each room. Measure every heated room here. This form records the inputs; it does not calculate the heat loss.

Rooms  (copy this block for each one)

Room *

______________________________________________________________________________

Length (m) *

______________________________________________________________________________

Width (m) *

______________________________________________________________________________

Ceiling height (m) *

______________________________________________________________________________

Floor area (m²)  (if applicable; worked out automatically in the app)

______________________________________________________________________________

Volume (m³)  (if applicable; worked out automatically in the app)

______________________________________________________________________________

Design temperature (°C)

______________________________________________________________________________

Number of external walls

______________________________________________________________________________

Window and door glazing area (m²)

______________________________________________________________________________

Heat emitter  (choose one)

☐ Radiator     ☐ Underfloor heating     ☐ Fan coil     ☐ None

Radiator size and type (for example 1200 x 600 double)  (if applicable)

______________________________________________________________________________

Photos  (photos: attach, or note the file names)

______________________________________________________________________________

Total heated floor area (m²)  (worked out automatically in the app)

______________________________________________________________________________

4. Outdoor unit, hot water and electrics

Proposed outdoor unit position *

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

Distance from the nearest boundary (m)

______________________________________________________________________________

Base, drainage for condensate and pipe route to the house

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

Are there neighbouring windows or gardens close to the unit position?

☐ Yes     ☐ No

Photos of the outdoor unit position  (photos: attach, or note the file names)

______________________________________________________________________________

Existing hot water cylinder size and location, if any

______________________________________________________________________________

Space for a new cylinder

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

Main fuse rating  (choose one)

☐ 60 A     ☐ 63 A     ☐ 80 A     ☐ 100 A     ☐ Not marked

Earthing arrangement  (choose one)

☐ TN-C-S (PME)     ☐ TN-S     ☐ TT     ☐ Not determined

Photos of the meter and consumer unit  (photos: attach, or note the file names)

______________________________________________________________________________

What the customer wants (comfort, running costs, timescale)

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

5. Actions and sign-off

Are further actions needed?

☐ Yes     ☐ No

Further actions  (copy this block for each one; if applicable)

What needs doing? *

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

Who will do it?

______________________________________________________________________________

Due date

______________________________________________________________________________

Status  (choose one)

☐ Open     ☐ In progress     ☐ Completed

Inspector name *

______________________________________________________________________________

Position or company

______________________________________________________________________________

Inspector signature

Signing records what was found on the day. It does not make unsafe equipment safe or replace a thorough examination, design check or service by a competent person.