Highway and Car Park Safety Inspection

A highway, footway and car park safety inspection: route, hierarchy and inspection type, each defect with its type, size, response category and photos, made safe, and actions.

1. Inspection

Project

______________________________________________________________________________

Client

______________________________________________________________________________

Site *

______________________________________________________________________________

Site address

______________________________________________________________________________

Inspection date *

______________________________________________________________________________

Time

______________________________________________________________________________

Inspector *

______________________________________________________________________________

Route, street or car park *

______________________________________________________________________________

Network hierarchy or category

______________________________________________________________________________

Inspection  (choose one)

☐ Routine (cyclic) safety inspection

☐ Reactive, after a report

☐ Night inspection (lighting)

Method  (choose one)

☐ Walked     ☐ Driven

Your investigatory level: carriageway or car park depth (mm)

Optional. From your own inspection policy. The code of practice leaves these levels to each authority or owner.

______________________________________________________________________________

Footway trip height (mm)

Optional. From your own inspection policy.

______________________________________________________________________________

For public highways in England and Wales, section 58 of the Highways Act 1980 gives the highway authority a defence to a claim if it took the care reasonably required, and the court considers whether it knew, or could reasonably have known, of the danger. The UK Roads Liaison Group code of practice says inspection records are crucial to defending claims, and that nil returns should be recorded too.

Weather capture

Digital form: use Record current conditions. On paper, record date/time, temperature (°C), wind (mph and direction), conditions and source. Keep observed conditions separate from forecast estimates.

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

2. Defects

Were any defects found?

☐ Yes     ☐ No

Defects  (copy this block for each one; if applicable)

Defect *  (choose one)

☐ Pothole

☐ Trip hazard on footway

☐ Kerb damage

☐ Ironwork or cover

☐ Signs

☐ Road markings

☐ Lighting

☐ Gully or drainage

☐ Vegetation or sight lines

☐ Barrier or bollard

☐ Other

Location (house number, bay, lamp column) *

______________________________________________________________________________

Where it is (GPS)

Optional. Mark the defect so the repair gang finds it.

______________________________________________________________________________

What is wrong?

______________________________________________________________________________

Depth or height (mm)

By a ruler or gauge at the deepest point or the highest edge.

______________________________________________________________________________

Width (mm)

Use a tape, or measure from a photo looking straight down, with a rule or a known bay line in the same plane.

______________________________________________________________________________

Length (mm)

______________________________________________________________________________

Area (m²)  (if applicable; worked out automatically in the app)

______________________________________________________________________________

Against your investigatory level  (if applicable; worked out automatically in the app)

______________________________________________________________________________

Response *  (choose one)

☐ Make safe within 24 hours

☐ Repair within 28 days

☐ Planned works

☐ Monitor at next inspection

Made safe on site (coned or signed)?  (if applicable)

☐ Yes     ☐ No

Photos (with a scale if possible)  (photos: attach, or note the file names)

______________________________________________________________________________

is_over  (worked out automatically in the app)

______________________________________________________________________________

is_unsafe  (worked out automatically in the app)

______________________________________________________________________________

is_area  (worked out automatically in the app)

______________________________________________________________________________

If applicable: Not made safe on site. Cone or sign it now, or arrange the response within 24 hours, and record who was told.

Defects needing action within 24 hours  (if applicable; worked out automatically in the app)

______________________________________________________________________________

Defects to repair within 28 days  (if applicable; worked out automatically in the app)

______________________________________________________________________________

Planned works  (if applicable; worked out automatically in the app)

______________________________________________________________________________

Monitor at the next inspection  (if applicable; worked out automatically in the app)

______________________________________________________________________________

At or over your investigatory level  (if applicable; worked out automatically in the app)

______________________________________________________________________________

24-hour defects not made safe  (if applicable; worked out automatically in the app)

______________________________________________________________________________

Area to repair (m²)  (if applicable; worked out automatically in the app)

______________________________________________________________________________

If applicable: A defect needing action within 24 hours was not made safe on site. Arrange it now and record who was told.

Well-managed Highway Infrastructure, the UK Roads Liaison Group code of practice, is not statutory. Its 2016 edition recommends a risk-based inspection regime, with each authority setting inspection frequencies by the network hierarchy and recording defects with a response based on risk. The response categories here are examples: use your own policy.

Walk Cam: route inspection

Walk Cam video walk-through: available in the SiteSurvey app only.

3. Actions and sign-off

Are further actions needed?

☐ Yes     ☐ No

Further actions  (copy this block for each one; if applicable)

What needs doing? *

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

Priority  (choose one)

☐ Urgent     ☐ High     ☐ Medium     ☐ Low

Who will do it?

______________________________________________________________________________

Due date

______________________________________________________________________________

Status  (choose one)

☐ Open     ☐ In progress     ☐ Completed

Actions open or in progress  (if applicable; worked out automatically in the app)

______________________________________________________________________________

Next inspection due

Set by the inspection frequency the authority or owner has set for this hierarchy category.

______________________________________________________________________________

Days to next due  (if applicable; worked out automatically in the app)

______________________________________________________________________________

If applicable: The next inspection due date is before the inspection date. Check the date.

Inspector name *

______________________________________________________________________________

Position or company

______________________________________________________________________________

Inspector signature

Signing records what was found on the day. It does not make unsafe equipment safe or replace a thorough examination, design check or service by a competent person.