Loft Insulation Pre-install Survey

A loft insulation pre-install survey: access, existing insulation depth, joists, ventilation and condensation, cables and downlights, tanks and pipes, boarding and storage, area and the work proposed.

1. Loft

Project

______________________________________________________________________________

Client

______________________________________________________________________________

Site *

______________________________________________________________________________

Site address

______________________________________________________________________________

Survey date *

______________________________________________________________________________

Time

______________________________________________________________________________

Surveyor *

______________________________________________________________________________

Safe access hatch and ladder?

☐ Yes     ☐ No

Loft length (m)

______________________________________________________________________________

Loft width (m)

______________________________________________________________________________

Loft area (m²)  (if applicable; worked out automatically in the app)

______________________________________________________________________________

Existing insulation depth (mm)

______________________________________________________________________________

Joist depth (mm)

______________________________________________________________________________

Existing insulation  (choose one)

☐ None     ☐ Mineral wool     ☐ Loose fill     ☐ Boards     ☐ Mixed

Walk Cam: loft survey

Walk Cam video walk-through: available in the SiteSurvey app only.

2. Checks

Ventilation at the eaves or ridge  (choose one)

☐ Clear     ☐ Blocked     ☐ None found

If applicable: Keep the eaves and ventilation paths clear when insulating, or the loft can suffer condensation.

Any damp, condensation, mould or leaks?

☐ Yes     ☐ No

If applicable: Fix damp and leaks before insulating.

Recessed downlights in the ceiling below?

☐ Yes     ☐ No

If applicable: Do not cover downlights unless they are rated for it; fit covers or keep insulation clear. Lift cables above the insulation where you can.

Cold water tank or pipes in the loft?

☐ Yes     ☐ No

If applicable: Leave the ceiling under the cold water tank uninsulated, and insulate the tank and pipes, so they do not freeze.

Is the loft boarded or used for storage?

☐ Yes     ☐ No

Any suspect asbestos (old loose fill, boards, flue pipes)?

☐ Yes     ☐ No

If applicable: HSE: loose-fill asbestos, a loose fluffy insulation that may be blue-grey or whitish, can be found in loft spaces, and only an HSE-licensed contractor may work on it. Do not disturb suspect material. Arrange an asbestos survey before any work.

Photos  (photos: attach, or note the file names)

______________________________________________________________________________

3. Proposal

Insulation proposed (material and depth)

______________________________________________________________________________

Is it funded under a government scheme?

☐ Yes     ☐ No

If applicable: Ofgem: installers on ECO4 must be TrustMark registered, and ECO4 measures follow the PAS 2035 retrofit process. Check the rules of the scheme funding this job.

Notes for the installer

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

Customer name

______________________________________________________________________________

Customer signature

Completed by *

______________________________________________________________________________

Signature