Pallet Racking Inspection

A pallet racking inspection: damage by aisle, bay and level with green, amber and red ratings, offload actions, general checks and the date of the last expert inspection.

1. Inspection details

Project

______________________________________________________________________________

Client

______________________________________________________________________________

Site *

______________________________________________________________________________

Site address

______________________________________________________________________________

Date of inspection *

______________________________________________________________________________

Time

______________________________________________________________________________

Warehouse, area or aisles inspected *

______________________________________________________________________________

Inspected by *

______________________________________________________________________________

Person responsible for racking safety (PRRS)

The named person who manages racking safety on site and sets how often it is inspected.

______________________________________________________________________________

Type of inspection *  (choose one)

☐ Regular in-house inspection (weekly or as set by the PRRS)

☐ After an impact or reported damage

☐ Expert inspection by a technically competent person

Date of the last expert inspection

______________________________________________________________________________

Days since the expert inspection  (if applicable; worked out automatically in the app)

______________________________________________________________________________

If applicable: More than 12 months since the expert inspection. HSE warehousing guidance (HSG76) says a technically competent person should inspect racking at intervals of not more than 12 months, alongside regular in-house inspections like this one.

2. Damage found

Damage categories (HSE HSG76, table 11, based on SEMA guidance). Green: within the limit, safe for further service; record it and look at it again at later inspections. Amber: hazardous damage needing action as soon as possible; once the load is removed, do not reload it until repaired. Amber damage not repaired within 4 weeks should be treated as red. Red: very serious damage; offload and isolate the area immediately until repaired.

Did you find any damage?

☐ Yes     ☐ No

Damage  (copy this block for each one; if applicable)

Aisle, bay and beam level *

______________________________________________________________________________

Component *  (choose one)

☐ Upright

☐ Beam

☐ Bracing (diagonal or horizontal)

☐ Base plate or floor fixing

☐ Beam connector or safety lock

☐ Decking or pallet support

☐ Column guard

☐ Other

What is the damage? *

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

Category *  (choose one)

☐ Green: record and watch     ☐ Amber: repair soon     ☐ Red: offload now

If applicable: Amber. Do not put new loads on this rack once it is emptied, and arrange a repair within 4 weeks.

Is it labelled so it is not reloaded until repaired?  (if applicable)

☐ Yes     ☐ No

If applicable: Red. Offload the affected racking now and keep it out of use until it has been repaired.

Has it been offloaded and isolated?  (if applicable)

☐ Yes     ☐ No

Repair by (who and date)  (if applicable)

______________________________________________________________________________

Photos  (photos: attach, or note the file names)

______________________________________________________________________________

Red items  (if applicable; worked out automatically in the app)

______________________________________________________________________________

Amber items  (if applicable; worked out automatically in the app)

______________________________________________________________________________

Green items  (if applicable; worked out automatically in the app)

______________________________________________________________________________

Walk Cam: racking damage visual inspection

Walk Cam video walk-through: available in the SiteSurvey app only.

3. General checks

Load notices displayed, readable and matching the configuration  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

Floor fixings in place, floor not cracked or uneven  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

Beam safety locks fitted on every beam  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

Column guards and end-of-aisle protectors in place and undamaged  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

Uprights plumb, no visible lean or bowing  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

Loads within the rated capacity, pallets sound and sitting squarely  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

Decking, mesh and pallet supports in place  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

Aisles clear, no items stored in flue spaces or on the floor in front  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

No unauthorised changes to beam levels or configuration  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

Photos  (photos: attach, or note the file names)

______________________________________________________________________________

4. Actions and sign-off

Overall result *  (choose one)

☐ No damage needing action

☐ Repairs needed (amber)

☐ Areas offloaded and isolated (red)

Summary for the PRRS

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

Are further actions needed?

☐ Yes     ☐ No

Further actions  (copy this block for each one; if applicable)

What needs doing? *

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

Who will do it?

______________________________________________________________________________

Due date

______________________________________________________________________________

Status  (choose one)

☐ Open     ☐ In progress     ☐ Completed

Inspector name *

______________________________________________________________________________

Position or company

______________________________________________________________________________

Inspector signature

Signing records what was found on the day. It does not make unsafe equipment safe or replace a thorough examination, design check or service by a competent person.