Rack Audit

Cabinet by cabinet audit: equipment by U position, power, cooling, labelling, cable management.

1. Location

Which cabinet, where, and what it looks like now.

Project

Filled in from the project in SiteSurvey.

______________________________________________________________________________

Site *

______________________________________________________________________________

Room or comms cupboard

______________________________________________________________________________

Cabinet ID *

______________________________________________________________________________

Rack size (U)

______________________________________________________________________________

Cabinet lock  (choose one)

☐ Locked     ☐ Lockable, found unlocked     ☐ No lock

Photos of the front, doors open  (photos: attach, or note the file names)

______________________________________________________________________________

Photos of the back  (photos: attach, or note the file names)

______________________________________________________________________________

2. Equipment

One entry for each item, working from the top of the cabinet down.

Most cabinets number U1 at the bottom. Check the numbers on the rails before you start and record the lowest and highest U an item takes, for example 38 to 39.

Equipment  (copy this block for each one)

U position

______________________________________________________________________________

Device  (choose one)

☐ Patch panel

☐ Fibre patch panel

☐ Network switch

☐ Router or firewall

☐ Server

☐ Storage or NAS

☐ NVR or DVR

☐ UPS

☐ PDU

☐ Shelf

☐ Cable manager or brush panel

☐ Blanking plate

☐ AV matrix or processor

☐ Amplifier

☐ Other: ____________

Make and model

______________________________________________________________________________

Labelled?

☐ Yes     ☐ No

Powered on?

☐ Yes     ☐ No

Items recorded  (worked out automatically in the app)

______________________________________________________________________________

Items with no label  (worked out automatically in the app)

______________________________________________________________________________

3. Power and cooling

How the cabinet is fed, and whether it is running cool.

Number of PDUs

______________________________________________________________________________

Is there a UPS?

☐ Yes     ☐ No

UPS status on the display  (if applicable; choose one)

☐ Normal     ☐ On battery     ☐ Alarm or fault showing

If applicable: The UPS is on battery or showing a fault. Tell the site contact now: the cabinet could lose power without warning.

Is dual-corded equipment fed from two different PDUs?

☐ Yes     ☐ No

Load notes

PDU readings, overloaded strips, daisy-chained extensions.

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

Temperature at the front of the cabinet (°C)

______________________________________________________________________________

If applicable: This is above 27 °C, the top of the ASHRAE recommended range for air going into IT equipment. Check the room cooling and anything blocking the airflow.

Cabinet fans  (choose one)

☐ Working     ☐ Not working     ☐ No fans fitted

Are empty U spaces fitted with blanking plates?

☐ Yes     ☐ No

4. Cabling

Patching, labels and cable management.

Is the patching tidy?

☐ Yes     ☐ No

Does the labelling meet the site standard?

☐ Yes     ☐ No

Cable management  (circle one (Poor to Very good))

☐ 1     ☐ 2     ☐ 3     ☐ 4     ☐ 5

Issues found  (tick all that apply)

☐ Unlabelled cables

☐ Over-length patch leads

☐ Cables blocking airflow

☐ Tight bends or crushed cables

☐ Loose or unsupported cables

☐ Redundant cables left in place

☐ Doors cannot close

☐ Power and data not separated

☐ Other: ____________

Photos of the issues  (if applicable; photos: attach, or note the file names)

______________________________________________________________________________

Action needed  (if applicable)

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

Auditor sign-off

Name

______________________________________________________________________________

Date

______________________________________________________________________________

Auditor signature