Utility Survey Record (PAS 128)

An underground utility survey record to PAS 128: survey type and quality level, records searched, site reconnaissance, detection methods, each utility found with depth, confidence and verification, and limitations.

1. Survey

Project

______________________________________________________________________________

Client

______________________________________________________________________________

Site *

______________________________________________________________________________

Site address

______________________________________________________________________________

Survey date *

______________________________________________________________________________

Time

______________________________________________________________________________

Surveyor *

______________________________________________________________________________

Company

______________________________________________________________________________

Survey types carried out *  (tick all that apply)

☐ Type D: desktop utility records search

☐ Type C: site reconnaissance

☐ Type B: detection

☐ Type A: verification

PAS 128:2022 (underground utility detection, verification and location) survey types and quality levels: D is a desktop search of existing records; C adds a site visit to look for visible evidence such as covers and trench scars; B detects utilities, mainly with electromagnetic location (EML) and ground penetrating radar (GPR); A verifies them by seeing them in a chamber or exposing them. QL-D gives the lowest confidence and QL-A the highest.

If applicable: TSA client guide to PAS 128: a Type D records search is mandatory for a Type A survey.

Survey area and extent *

______________________________________________________________________________

Walk Cam: Type C site reconnaissance

Walk Cam video walk-through: available in the SiteSurvey app only.

Weather capture

Digital form: use Record current conditions. On paper, record date/time, temperature (°C), wind (mph and direction), conditions and source. Keep observed conditions separate from forecast estimates.

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

2. Records and reconnaissance

Utility records obtained  (tick all that apply)

☐ Electricity

☐ Gas

☐ Water

☐ Telecoms

☐ Foul or surface water drainage

☐ Street lighting

☐ Fibre

☐ None obtained

Visible evidence found  (tick all that apply)

☐ Manhole or chamber covers

☐ Valve or stop tap covers

☐ Street lighting columns

☐ Telecoms cabinets

☐ Meters

☐ Reinstatement scars

☐ Marker posts

Detection methods  (tick all that apply)

☐ EML (cable locator and signal generator)

☐ GPR

☐ Sonde

☐ Chamber inspection

☐ Trial holes or vacuum excavation

Equipment and calibration

______________________________________________________________________________

Site photos and marked-out services  (photos: attach, or note the file names)

______________________________________________________________________________

3. Utilities found

Utilities  (copy this block for each one)

Utility *  (choose one)

☐ Electricity

☐ Gas

☐ Water

☐ Telecoms

☐ Foul or surface water drainage

☐ Street lighting

☐ Fibre

☐ Unknown

Location or line reference

______________________________________________________________________________

Line positions (GPS)

Walk the marked line and Mark current location at each end and at changes of direction. Phone GPS is good to a few metres: the marks on the ground and the survey drawing are the record to dig by.

______________________________________________________________________________

Length traced (m)

______________________________________________________________________________

Depth to top (m)

From the surface to the top of the service. In a trial hole, photograph it square on with a staff or rod standing on the service and use a marked length as the reference. A locator or GPR depth is an estimate: say how it was found.

______________________________________________________________________________

Depth found by  (choose one)

☐ Trial hole or chamber (seen)

☐ Locator estimate

☐ GPR estimate

☐ Records only

☐ Not found

Quality level *  (choose one)

☐ QL-D     ☐ QL-C     ☐ QL-B4     ☐ QL-B3     ☐ QL-B2     ☐ QL-B1     ☐ QL-A

Size and material, if known

______________________________________________________________________________

Marked on site?

☐ Yes     ☐ No

Photos  (photos: attach, or note the file names)

______________________________________________________________________________

Utilities recorded  (worked out automatically in the app)

______________________________________________________________________________

Length traced (m)  (worked out automatically in the app)

______________________________________________________________________________

Utilities verified (QL-A)  (worked out automatically in the app)

______________________________________________________________________________

Detected (QL-B1 to B4)  (worked out automatically in the app)

______________________________________________________________________________

From site evidence (QL-C)  (worked out automatically in the app)

______________________________________________________________________________

From records only (QL-D)  (worked out automatically in the app)

______________________________________________________________________________

Unknown utilities  (worked out automatically in the app)

______________________________________________________________________________

Without a depth  (worked out automatically in the app)

______________________________________________________________________________

Not marked on site  (worked out automatically in the app)

______________________________________________________________________________

If applicable: Unknown, unmarked or depthless utilities are recorded. Pass them on to whoever plans the dig, and treat them as live until proved otherwise.

Since PAS 128:2022, QL-B is split into B1 to B4 by how well the position and depth were found. For example, a utility located but whose depth could not be found is QL-B3; one located with a well-defined position and depth is QL-B1. A P suffix shows GPR data was post-processed.

Limitations (areas not surveyed, poor ground conditions, congested services)

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

Survey drawing or deliverable reference

______________________________________________________________________________

No survey method can guarantee to find every buried utility. HSE HSG47: plans alone are not enough to locate services; use locators often during the work, dig trial holes by hand or by vacuum excavation to confirm detected services, and if in doubt treat services as live.

4. Sign-off

Client name

______________________________________________________________________________

Client signature

Completed by *

______________________________________________________________________________

Signature