Vehicle Inspection Checklist

A daily vehicle inspection checklist and defect report for vans and cars: walkaround checks, mileage, tyre tread, photos of each side, defects graded safe or not to drive, and fleet sign-off.

1. Vehicle and driver

Project

______________________________________________________________________________

Client

______________________________________________________________________________

Depot or site

______________________________________________________________________________

Depot or site address

______________________________________________________________________________

Date of the check *

______________________________________________________________________________

Time

______________________________________________________________________________

Driver *

______________________________________________________________________________

Registration number *

______________________________________________________________________________

Scan the number plate or fleet tag

Scanned in the SiteSurvey app. On paper, write the numbers: registration, fleet number.

______________________________________________________________________________

______________________________________________________________________________

Vehicle  (choose one)

☐ Small van     ☐ Panel van (up to 3.5 tonnes)     ☐ Pickup     ☐ Car     ☐ Other

Make and model

______________________________________________________________________________

Odometer reading *

______________________________________________________________________________

Fuel or charge level  (choose one)

☐ Full     ☐ 3/4     ☐ 1/2     ☐ 1/4     ☐ Reserve

DVSA van walkaround check. These are the 19 checks in the DVSA van drivers' daily walkaround check. Record the registration, the date, each defect, your assessment of it, your name and who you reported it to. Record nil defects if you find none. DVSA can ask for the record at a roadside check.

2. Walkaround checks

1. Front view, mirrors, cameras and glass: nothing blocking the view, windscreen not damaged  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

2. Windscreen washers and wipers work and are in good condition  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

3. Dashboard warning lights working, including ABS, airbag (SRS), EBS, EPS, ESC, full beam and parking brake  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

4. Steering moves properly, no excessive play  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

5. Horn works  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

6. Footwell clear; service brake and parking brake work without excessive travel  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

7. Seats secure; seat belts undamaged, latch and retract  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

8. Lights and indicators work; lenses fitted, clean and the right colour  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

9. Fluids, fuel and oil: levels correct, fuel filler cap secure, no leaks or puddles  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

10. Battery secure, in good condition, not leaking  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

11. Bodywork and doors: doors shut and secure, no sharp edges or loose panels  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

12. Tyres and wheels: legal tread, inflated, no deep cuts or visible cord, wheels secure  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

13. Load secure, right securing system, doors and tailgate lock  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

14. AdBlue (diesel exhaust fluid) topped up  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

15. Exhaust not leaking, no excessive smoke or noise  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

16. Alternative fuel system: isolation switch located, no leaks  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

17. Number plates correct, secure and readable  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

18. Tow bar and trailer secure, electrics working, breakaway cable fitted  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

19. Other equipment, such as a tail lift or other loading equipment, secure, undamaged and working  (choose one)

☐ OK     ☐ Defect     ☐ Not applicable

What is wrong?  (if applicable)

______________________________________________________________________________

Lowest tread depth measured (mm)

______________________________________________________________________________

If applicable: Below the legal minimum. Cars and light vans need at least 1.6 mm of tread across the central three-quarters of the tread and around the whole circumference (Highway Code, annex 6). You can get 3 penalty points for each tyre that is not safe and legal. Do not drive the vehicle until the tyre is replaced.

Photos of the front, back and both sides  (photos: attach, or note the file names)

Four photos show the vehicle's condition at the start of the shift.

______________________________________________________________________________

3. Defects

Did you find any defects?

Answer No to record nil defects.

☐ Yes     ☐ No

Defects  (copy this block for each one; if applicable)

What is the defect and where? *

______________________________________________________________________________

Can the vehicle be driven? *  (choose one)

☐ Minor: safe to drive, report for repair     ☐ Dangerous: do not drive

If applicable: Do not drive. DVSA says dangerous defects must be fixed before the journey continues. Tell your supervisor or fleet manager now.

Photos  (photos: attach, or note the file names)

______________________________________________________________________________

Reported to

______________________________________________________________________________

Dangerous defects  (if applicable; worked out automatically in the app)

______________________________________________________________________________

Result *  (choose one)

☐ Nil defects

☐ Minor defects reported, safe to drive

☐ Dangerous defect: do not drive

4. Sign-off

Driver name *

______________________________________________________________________________

Driver signature

Fleet manager or supervisor  (if applicable)

______________________________________________________________________________

Repair carried out and by whom  (if applicable)

______________________________________________________________________________

______________________________________________________________________________

______________________________________________________________________________

Date repaired  (if applicable)

______________________________________________________________________________

If applicable: The repair date is before the date of the check. Check both dates.

Supervisor signature  (if applicable)